Experiment Number: 1

Simulation and Comparative Analysis of FDMA, TDMA and CDMA: Resource Allocation, User Capacity and Mobile-Generation Context Program 1 — Capacity estimate for FDMA, TDMA and CDMA Objective 1: To estimate and compare the illustrative number of users supported by FDMA,

TDMA and CDMA using the specified system parameters. CODE:

clear; clc; close all;

% Illustrative system parameters

totalBandwidthHz = 1e6; % Total bandwidth: 1 MHz fdmaChannelHz = 200e3; % FDMA channel width: 200 kHz tdmaSlots = 8; % Number of TDMA slots per frame processingGain = 64; % CDMA processing gain targetSIRdB = 7; % Minimum acceptable CDMA SIR

% Capacity estimates under the stated assumptions fdmaUsers = floor(totalBandwidthHz / fdmaChannelHz); tdmaUsers = tdmaSlots;

targetSIR = 10^(targetSIRdB / 10);

cdmaUsers = floor(1 + processingGain / targetSIR); fprintf('Illustrative maximum supported users\n'); fprintf('FDMA: %d users\n', fdmaUsers); fprintf('TDMA: %d users\n', tdmaUsers);

fprintf('CDMA: %d users at a target SIR of %.1f dB\n\n', ... cdmaUsers, targetSIRdB);

% CDMA SIR as the number of simultaneous users increases K = 2:30;

sirLinear = processingGain ./ (K - 1); sirDb = 10 * log10(sirLinear);

% Display a comparison table

method = ["FDMA"; "TDMA"; "CDMA"];

supportedUsers = [fdmaUsers; tdmaUsers; cdmaUsers];

results = table(method, supportedUsers); disp(results);

% Plot estimated user capacity figure;

bar(categorical(method), supportedUsers);

ylabel('Illustrative maximum supported simultaneous users'); title('FDMA, TDMA and CDMA: Capacity Estimate');

grid on;

% Plot estimated CDMA SIR as users are added figure;

plot(K, sirDb, 'o-', 'LineWidth', 1.5); hold on;

yline(targetSIRdB, '--r', 'Target SIR'); xlabel('Number of simultaneous CDMA users (K)'); ylabel('Estimated SIR (dB)');

title('Estimated CDMA SIR versus Number of Users'); grid on;

hold off;

# OUTPUT:

Objective 2: CDMA SIR analysis:

To analyze how the estimated CDMA signal-to-interference ratio changes as the number of simultaneous users increases.

Objective 3: Mobile-generation comparison:

To relate representative multiple-access techniques to the evolution of mobile communication

systems from 1G to 5G. CODE:

generation = ["1G"; "2G"; "2G"; "3G"; "4G"; "5G"];

system = ["AMPS"; "GSM"; "IS-95"; "UMTS"; "LTE"; "5G NR"];

accessMethod = ["FDMA"; "TDMA"; "CDMA"; "WCDMA"; "OFDMA"; "OFDMA-based"];

keyIdea = ["Separate frequency channels"; ... "Separate time slots"; ...

"Code-based user separation"; ... "Wideband code-based access"; ... "Orthogonal frequency resources"; ... "Flexible OFDM-based radio access"];

evolutionTable = table(generation, system, accessMethod, keyIdea); disp(evolutionTable);

# OUTPUT:

Objective 4: Resource-allocation diagrams:

To visualize how FDMA, TDMA and CDMA assign frequency, time and code resources to

multiple users. CODE:

figure; tiledlayout(1,3);

% FDMA: users occupy different frequency channels nexttile;

hold on;

colors = lines(4); for u = 1:4

rectangle('Position',[0, u-1, 1, 0.8], ...

'FaceColor',colors(u,:), 'EdgeColor','k');

text(0.5, u-0.6, sprintf('User %d',u), ... 'HorizontalAlignment','center');

end

xlim([0 1]); ylim([0 4]); xlabel('Time'); ylabel('Frequency channel'); title('FDMA');

set(gca,'YTick',[]); hold off;

% TDMA: users occupy different time slots on one frequency channel nexttile;

hold on; for u = 1:4

rectangle('Position',[u-1, 0, 0.8, 1], ...

'FaceColor',colors(u,:), 'EdgeColor','k');

text(u-0.6, 0.5, sprintf('User %d',u), ... 'HorizontalAlignment','center');

end

xlim([0 4]); ylim([0 1]); xlabel('Time slot'); ylabel('Frequency channel'); title('TDMA'); set(gca,'YTick',[]);

hold off;

% CDMA: users share the same time and frequency band using different codes nexttile;

hold on; for u = 1:4

rectangle('Position',[0, 0, 1, 1], ...

'FaceColor',colors(u,:), 'FaceAlpha',0.22, ...

'EdgeColor',colors(u,:), 'LineWidth',1.5); end

text(0.5, 0.62, 'Users 1-4 share', 'HorizontalAlignment','center'); text(0.5, 0.40, 'band; different codes', 'HorizontalAlignment','center'); xlim([0 1]); ylim([0 1]);

xlabel('Time'); ylabel('Frequency band'); title('CDMA'); set(gca,'YTick',[]);

hold off;

sgtitle('Conceptual Time-Frequency Resource Allocation');

# OUTPUT:

Experiment Number: 2

MATLAB Simulation of Cellular Coverage, Cell Splitting and Handoff in a Multi-Cell

Network CODE:

% Experiment 2: simplified hexagonal coverage and handoff clear; clc; close all;

% Baseline assumptions serviceAreaKm2 = 5;

radiiKm = [1.0 0.5];

usersPerCell = 30;

hexArea = @(R) (3*sqrt(3)/2) .* R.^2; cellAreaKm2 = hexArea(radiiKm);

cellCount = ceil(serviceAreaKm2 ./ cellAreaKm2); estimatedUsers = cellCount .* usersPerCell;

capacityTable = table(radiiKm(:), cellAreaKm2(:), cellCount(:), ... repmat(usersPerCell,numel(radiiKm),1), estimatedUsers(:), ... 'VariableNames', {'Radius_km','CellArea_km2','EstimatedCells', ... 'UsersPerCell','IllustrativeUsers'});

disp(capacityTable);

% Plot a 7-cell cluster: central cell plus six neighbors R = radiiKm(1);

centers = [0 0; 1.5*R sqrt(3)*R/2; 1.5*R -sqrt(3)*R/2; ... 0 sqrt(3)*R; 0 -sqrt(3)*R; -1.5*R sqrt(3)*R/2; ...

-1.5*R -sqrt(3)*R/2];

theta = (0:6)*pi/3; % flat-top hexagon vertices figure; hold on; axis equal; grid on;

for k = 1:size(centers,1)

plot(centers(k,1)+R*cos(theta), centers(k,2)+R*sin(theta), 'k-'); text(centers(k,1),centers(k,2),sprintf('Cell %d',k), ... 'HorizontalAlignment','center');

end

xlabel('x (km)'); ylabel('y (km)'); title('Idealized Seven-Cell Hexagonal Cluster');

hold off;

% Mobile moves across the boundary between centers at x=0 and x=1.5R mobileX = linspace(-0.2*R,1.7*R,300);

mobileY = zeros(size(mobileX)); d1 = hypot(mobileX, mobileY);

d2 = hypot(mobileX-1.5*R, mobileY); servingCell = ones(size(mobileX)); servingCell(d2 < d1) = 2;

handoffIndex = find(diff(servingCell) ~= 0,1,'first') + 1; figure; hold on; grid on; plot(mobileX,d1,'LineWidth',1.5); plot(mobileX,d2,'LineWidth',1.5);

if ~isempty(handoffIndex) xline(mobileX(handoffIndex),'--r','Handoff decision'); end

xlabel('Mobile x-position (km)'); ylabel('Distance to cell center (km)'); legend('Cell 1','Cell 2','Location','best');

title('Nearest-Center Handoff Approximation'); hold off;

figure; stairs(mobileX,servingCell,'LineWidth',1.5); grid on; xlabel('Mobile x-position (km)'); ylabel('Serving-cell ID'); title('Serving Cell Along the Mobile Path');

if ~isempty(handoffIndex)

fprintf('Geometric handoff at x = %.3f km\n',mobileX(handoffIndex)); end

# OUTPUT:

Experiment Number: 3

MATLAB Analysis of Spectrum Bands, Propagation Loss and Cellular Interference

# CODE:

% Experiment 3: illustrative propagation and frequency reuse clear; clc; close all;

% Representative bands and link-budget assumptions freqGHz = [0.7 3.5 28];

freqMHz = freqGHz * 1000;

bandName = ["Sub-1 GHz"; "Mid-band"; "mmWave"]; txPowerdBm = 30;

gtDbi = 0; grDbi = 0;

distanceKm = logspace(log10(0.1),log10(2),200); distanceAtPointKm = 0.5;

% Free-space path loss (d in km, f in MHz)

fspl = @(fMHz,dKm) 32.44 + 20*log10(fMHz) + 20*log10(dKm); lossAtPoint = fspl(freqMHz,distanceAtPointKm);

rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint;

linkTable = table(bandName,freqGHz(:),lossAtPoint(:),rxAtPoint(:), ... 'VariableNames',{'Band','Frequency_GHz','FSPL_dB','EstimatedRx_dBm'}); disp(linkTable);

% Path loss and received power over distance figure; tiledlayout(1,2);

nexttile; hold on; grid on; for k=1:numel(freqMHz)

plot(distanceKm,fspl(freqMHz(k),distanceKm),'LineWidth',1.5); end

set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Path loss (dB)'); legend(bandName,'Location','northwest'); title('Free-Space Path Loss'); nexttile; hold on; grid on;

for k=1:numel(freqMHz)

rx = txPowerdBm + gtDbi + grDbi - fspl(freqMHz(k),distanceKm); plot(distanceKm,rx,'LineWidth',1.5);

end

set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Estimated received power (dBm)');

legend(bandName,'Location','southwest'); title('Illustrative Received Power');

% Simple first-tier co-channel reuse estimate cellRadiusKm = 0.5;

reuseN = [1 3 7 12];

pathLossExponent = 3.5;

reuseDistance = sqrt(3*reuseN) * cellRadiusKm;

sirLinear = (reuseDistance/cellRadiusKm).^pathLossExponent / 6; sirDb = 10*log10(sirLinear);

reuseTable = table(reuseN(:),reuseDistance(:),sirDb(:), ... 'VariableNames',{'ReuseClusterN','ApproxInterfererDistance_km','ApproxSIR_dB'}); disp(reuseTable);

figure; plot(reuseN,sirDb,'o-','LineWidth',1.5); grid on; xlabel('Reuse cluster size N'); ylabel('Approximate SIR (dB)'); title('Simplified Co-Channel SIR versus Reuse Cluster Size');

fprintf('At %.2f km, estimated received powers are:\n',distanceAtPointKm); disp(table(bandName,rxAtPoint(:),'VariableNames',{'Band','Rx_dBm'}));

# OUTPUT: